Digital Egypt Pioneers Initiative (DEPI)

Electricity Demand Monitor

Literature Review

Electricity demand and weather data sources, existing open-source projects, and data engineering implications for a Python / SQL Server monitoring system

Project: Electricity Demand Monitor (DEPI Graduation Project)

Team / Student Name(s):

Name

Ahmed Samy Aboelftooh. Ahmed Abdalshakor Ramzy. Youssif Nasser Abd Elaziz. Hossam Mohammed Asim. Yomna Samir Abdelaziz. Adel Mohmed Elsayd.

Table of Contents

1. Introduction	1

1.1 Electricity demand and load	1

1.2 Weather as analytical context	1

1.3 Monitoring versus forecasting	1

1.4 Structure of this review	1

2. Existing Real-World Electricity Data Platforms	1

2.1 U.S. Energy Information Administration (EIA)	1

2.2 ENTSO-E Transparency Platform	1

2.3 Red Eléctrica / Redeia REData API	1

2.4 Electricity Maps	1

2.5 Summary	1

3. Weather Data Sources	1

3.1 Open-Meteo	1

3.2 NOAA / NCEI	1

3.3 Aligning weather with electricity demand	1

4. Real GitHub Projects	1

5. Electricity Demand Forecasting (Optional Extension)	1

5.1 What it is	1

5.2 Why historical demand and weather are useful	1

5.3 Common approaches at a high level	1

5.4 Possible integration into the project	1

6. Data Engineering Implications	1

7. Research Gap / Project Relevance	1

8. Comparison Table	1

9. Proposed Project Context	1

10. Conclusion	1

References	1

# 1. Introduction

## 1.1 Electricity demand and load

Electricity demand, also called electric load, is the amount of electric power or energy that consumers draw from the grid over time. It is typically reported in megawatts (MW) or megawatt-hours (MWh) at hourly or sub-hourly resolution for a country, balancing area, or market zone. Because electricity cannot be stored economically at large scale, system operators must continuously balance supply with demand, and analysts, planners, and researchers rely on historical and near-real-time load series to understand daily, weekly, and seasonal consumption patterns.

Monitoring electricity demand therefore means collecting load observations at regular intervals, validating them, storing them in a consistent structure, and presenting them in a form that supports interpretation, such as trend charts, peak-load views, and comparisons across periods.

## 1.2 Weather as analytical context

Weather, and temperature in particular, is a widely used explanatory context for load because heating and cooling needs change with outdoor conditions. Open-source projects reviewed in this document reflect this assumption: GridCast describes load as tightly coupled to temperature (Nzouh, n.d.), and Peaky Finders explores how well a model can perform using only historical load and temperature data (Baranko, 2020). For a monitoring system, placing temperature next to demand makes unusual consumption patterns easier to interpret.

## 1.3 Monitoring versus forecasting

Monitoring describes what has happened or is currently happening: it focuses on collecting, validating, storing, and visualizing observed data. Forecasting estimates future values, usually by learning relationships between past demand and explanatory variables such as time of day, day of week, season, and temperature (Hong & Fan, 2016; Hyndman & Athanasopoulos, 2021). The two activities are related, since both depend on the same clean historical dataset, but they differ in purpose and complexity.

The initial project scope focuses on electricity demand monitoring, weather comparison, data processing, and dashboard visualization. Forecasting may be considered as an additional analytical feature depending on project requirements, data availability, and implementation feasibility. This review therefore covers forecasting as an open, optional extension rather than a confirmed component.

## 1.4 Structure of this review

Section 2 reviews electricity data platforms and Section 3 reviews weather data services. Section 4 examines verified GitHub projects, and Section 5 discusses optional demand forecasting. Section 6 covers data engineering implications, Section 7 identifies the project relevance, Section 8 compares the reviewed work, Section 9 describes the proposed project, and Section 10 concludes.

# 2. Existing Real-World Electricity Data Platforms

Several official organizations publish electricity data that could serve as sources for the project. The descriptions below are limited to what is stated in their official documentation; access conditions, coverage, and terms of use should be rechecked before implementation.

## 2.1 U.S. Energy Information Administration (EIA)

EIA is the statistical and analytical agency of the U.S. Department of Energy. It provides programmatic access to its datasets through the EIA Open Data service (https://www.eia.gov/opendata/) and publishes hourly electric system operating data through the Hourly Electric Grid Monitor (https://www.eia.gov/electricity/gridmonitor/). Relevance: it is a candidate source of U.S. hourly demand by balancing authority or region, and the GridCast project reviewed below lists EIA among its data pulls (Nzouh, n.d.). Registration for an API key is required for the Open Data service.

## 2.2 ENTSO-E Transparency Platform

The ENTSO-E Transparency Platform (https://transparency.entsoe.eu/) is operated by the European Network of Transmission System Operators for Electricity and publishes European electricity market and grid data, including load data. Relevance: it offers a recognized source of country- and zone-level load for Europe. Programmatic access follows the platform's own API guidance and requires registration, so it would need to be reviewed against project time constraints. The dataset catalog by LSB-dev (n.d.) also lists an ENTSO-E electric load dataset among publicly available load-forecasting datasets.

## 2.3 Red Electrical / Redeia REData API

Red Eléctrica (Redeia) provides the REData API (https://www.ree.es/en/apidatos), described in its documentation as a REST service that gives third parties access to the data behind the REData application. Only GET requests are allowed, responses are returned in JSON following the JSON:API standard, and requests are filtered with parameters such as start date, end date, Timetric, geo_trunc, and geo_ids. The documentation lists demand-related widgets including demanda, demanda-maxima-diaria, demand-maxima-herbaria, variacion-demanda, and demand-Tiempo-real. Relevance: its simple, documented request format and geographic identifiers make it a practical candidate for a first ETL source for Spanish electricity demand.

## 2.4 Electricity Maps

Electricity Maps (https://www.electricitymaps.com/) provides a platform and data services about electricity grids, with emphasis on the carbon intensity and the generation mix of electricity by zone. Relevance: it is useful as a related reference for how grid data can be presented visually, but its emphasis is on carbon and generation mix rather than on raw demand/load monitoring, so it would be a secondary rather than a primary source. Its data access plans and terms should be checked on the official site.

## 2.5 Summary

EIA, ENTSO-E, and REData each provide official or operator-level demand-related data with documented access routes, but they differ in geography, resolution, authentication, and response format. These differences are the practical reason a monitoring project needs its own normalization layer, discussed in Section 6.

# 3. Weather Data Sources

## 3.1 Open-Meteo

Open-Meteo (https://open-meteo.com/) is an open weather data service with documented forecast and historical weather APIs (https://open-meteo.com/en/docs/historical-weather-api) that are queried by latitude and longitude and return hourly variables such as temperature in JSON. Relevance: coordinate-based queries make it straightforward to retrieve hourly temperature for a city or region. The GridCast repository states that its weather data comes from Open-Meteo (Nzouh, n.d.). Licensing and usage limits should be confirmed in the official terms.

## 3.2 NOAA / NCEI

The National Centers for Environmental Information (NCEI), part of the U.S. National Oceanic and Atmospheric Administration (NOAA), archives and distributes official U.S. and global climate and weather observations (https://www.ncei.noaa.gov/), including web services for Climate Data Online. Relevance: station-based observations from an authoritative institution are valuable when measured rather than modeled temperature is preferred, although station selection and coverage must be handled explicitly.

## 3.3 Aligning weather with electricity demand

Demand and weather come from different providers with different conventions, so alignment must be explicit:

- Timestamp: convert both datasets to a common time standard (preferably UTC, with local time derived afterwards) and a common interval, for example hourly.

- Location: map each demand region (country, balancing area, or zone) to one or more representative coordinates or stations. A single point is a simplification of a large area and should be documented as a limitation.

- Temperature: use temperature as the primary comparison variable, and optionally derive heating and cooling degree-day style indicators.

- Other variables: humidity, wind speed, cloud cover, solar radiation, and precipitation may be retained when the chosen API supplies them and they add analytical value.

# 4. Real GitHub Projects

Repositories were located through GitHub topic pages and repository pages and retained only if their purpose, technologies, and data sources could be read from the repository description or README at the time of access. Items that could not be confirmed are marked as not verified rather than assumed.

4.1 Peaky Finders

| Item | Details |
| --- | --- |
| Repository | kbaranko/peaky-finders — https://github.com/kbaranko/peaky-finders |
| Main purpose | A Plotly Dash application with peak-load visualizations and a day-ahead forecasting model for five ISOs; it states that it illustrates core concepts rather than cutting-edge methods. |
| Technologies | Plotly Dash; Pyiso library for ISO data; Dark Sky API for weather. |
| Data sources | Historical load collected through Pyiso from ISO websites; historical temperature by latitude/longitude from Dark Sky. |
| Similarity | Combines historical load with temperature and presents results in an interactive dashboard. |
| Difference | Uses different data sources and focuses on peak-load and day-ahead forecasting; no SQL Server storage is mentioned in the sources reviewed. |
| Forecasting | Yes (day-ahead model). |

4.2 GridCast

| Item | Details |
| --- | --- |
| Repository | nzouh/GridCast — https://github.com/nzouh/GridCast |
| Main purpose | Probabilistic 10-day grid-stress forecasting for AI data-centre operators, with a dashboard. |
| Technologies | Temporal Fusion Transformer built on PyTorch Forecasting; GFS weather ensemble; dashboard. |
| Data sources | EIA, ERCOT, and weather data from Open-Meteo (data/ folder described as data pulls and feature building). |
| Similarity | Uses EIA demand data with Open-Meteo weather and a dashboard layer; a data-pull stage precedes modeling. |
| Difference | Forecasting is the core objective; the project targets grid stress for data-centre operators rather than general demand monitoring. |
| Forecasting | Yes (core feature), benchmarked against a 24-hour persistence forecast. |

4.3 DemandCast

| Item | Details |
| --- | --- |
| Repository | open-energy-transition/demandcast — https://github.com/open-energy-transition/demandcast |
| Main purpose | Python-based collection, processing, and forecasting of hourly electricity demand data (global hourly demand forecasting). |
| Technologies | Python; ETL scripts; machine learning models; MkDocs documentation; GitHub Actions. |
| Data sources | Open hourly and sub-hourly demand data from public sources, plus weather and socio-economic data. |
| Similarity | Explicit ETL stage for demand and weather data; its documentation lists adding quality checks of demand time series as a development goal. |
| Difference | Global scope and a machine learning focus; dashboard and database storage were not verified. |
| Forecasting | Yes (machine learning models folder). |

4.4 Forecasting Electricity Weather

| Item | Details |
| --- | --- |
| Repository | alcheng10/Forecasting_Electricity_Weather — https://github.com/alcheng10/Forecasting_Electricity_Weather |
| Main purpose | Three forecasting works related to weather and electricity, with interactive versions hosted on Kaggle. |
| Technologies | Not specified in the README; the work is presented as Kaggle notebooks. |
| Data sources | Not verified in detail; notebook titles refer to electricity load data analysis and PJME (U.S.) electricity usage. |
| Similarity | Combines electricity load and weather analysis. |
| Difference | Notebook-based analysis; no collection pipeline, database, or dashboard was verified. |
| Forecasting | Yes (forecasting notebooks). |

A further resource, the Publicly Available Datasets for Electric Load Forecasting repository (LSB-dev, n.d., https://github.com/LSB-dev/Publicly-Available-Datasets-For-Electric-Load-Forecasting), is a dataset catalog rather than a monitoring system and is cited only as a source-discovery aid.

Synthesis. The reviewed repositories mostly emphasize forecasting; the collection and quality stages appear mainly as preparation for modeling, and a relational database such as SQL Server is not described in any of them. A monitoring-first system with an explicit storage and validation layer is therefore a reasonable and distinct educational scope.

# 5. Electricity Demand Forecasting (Optional Extension)

## 5.1 What it is

Electricity demand forecasting estimates future load over horizons ranging from hours to years. Hong and Fan (2016) review the field, distinguishing horizons and highlighting probabilistic forecasting as an important direction. Earlier reviews, such as Hippert et al. (2001), examined neural network approaches for short-term load forecasting.

## 5.2 Why historical demand and weather are useful

Load typically shows strong repeating structure (hour of day, day of week, season) and sensitivity to temperature. Historical demand captures the repeating structure, while weather variables explain part of the remaining variation. The Peaky Finders project uses features such as the previous day's load and temperature (Baranko, 2020).

## 5.3 Common approaches at a high level

- Simple baselines, such as naive or seasonal-naive forecasts (Hyndman & Athanasopoulos, 2021). GridCast benchmarks its model against a 24-hour persistence baseline (Nzouh, n.d.).

- Statistical time-series and regression models that use calendar and weather features.

- Machine learning and deep learning models, as in the repositories reviewed above.

## 5.4 Possible integration into the project

If sufficient data and implementation time are available, forecasting could be added as an optional analytical extension. Possible inputs include historical demand, temperature, hour, day of week, holidays, and seasonality. The cleaned SQL Server tables could supply the training data, forecast results could be written to a separate table, and the dashboard could overlay forecasts on observed values. A simple baseline and a time-ordered train/test split would be a reasonable starting point; no specific algorithm is committed to in this document.

# 6. Data Engineering Implications

The practical pipeline for the project can be summarized as:

Collect → Validate → Clean → Transform → Align → Store → Analyze → Visualize → Optional Forecast

Each stage addresses specific data risks that are visible in the sources reviewed:

- Missing values: gaps in hourly series must be detected and either flagged or filled by a documented rule.

- Duplicates: repeated API pulls can create duplicate timestamp–region records; unique keys in SQL Server help prevent this.

- Invalid timestamps: malformed, out-of-range, or non-existent local times (including daylight-saving transitions) need validation.

- Time zones: store timestamps in UTC and keep the original zone and local time as separate fields.

- Geographic identifiers: sources use different identifiers, such as REData's geo_ids; a mapping table should link demand regions to weather locations.

- Unit consistency: MW versus MWh, and °C versus °F or K, must be standardized.

- Data quality and source reliability: prefer official sources, record collection time and API version, and apply basic range and completeness checks.

- Traceability: retain source name, URL or endpoint, retrieval timestamp, and ETL run identifier so each record can be traced back to its origin.

# 7. Research Gap / Project Relevance

Existing platforms such as EIA, ENTSO-E, and REData provide electricity data, and services such as Open-Meteo and NOAA/NCEI provide weather data. The GitHub projects reviewed demonstrate analysis, dashboards, and forecasting, mostly built around modeling. What is less visible in the reviewed material is a compact, educational end-to-end workflow that connects the stages together.

The proposed project aims to integrate selected sources into a practical educational workflow that collects data, cleans data, aligns demand with weather, stores it in SQL Server, provides analysis and visualization, and may optionally extend to forecasting. The project does not claim to introduce a new scientific forecasting method; its contribution is practical integration and clear data engineering practice.

# 8. Comparison Table

Table 1 compares the reviewed platforms and projects. Entries are limited to information confirmed from the sources reviewed; "Not verified" indicates that it could not be confirmed.

| Project / Platform | Data Type | Electricity Demand | Weather | API / Data Access | Dashboard | Forecasting | Technologies | Relevance to Our Project |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EIA Open Data / Grid Monitor | Official U.S. energy statistics | Yes (U.S. grid data) | No | Open Data API (key required); web tools | Grid Monitor web tool | Not verified | Not verified | Candidate U.S. demand source |
| ENTSO-E Transparency Platform | European market and grid data | Yes (load) | No | Platform and API (registration) | Web platform | Not verified | Not verified | Candidate European load source |
| Redeia REData API | Spanish electricity data | Yes (demand widgets) | No | REST, GET, JSON (JSON:API) | REData application | Not verified | REST/JSON | Simple candidate ETL source |
| Electricity Maps | Grid carbon intensity and generation mix | Not the main focus | Not verified | Platform and API (see site terms) | Web map | Not verified | Not verified | Reference for presentation; secondary source |
| Open-Meteo | Weather forecast and history | No | Yes | REST API by latitude/longitude, JSON | No | No | REST/JSON | Candidate weather source |
| NOAA / NCEI | Official climate and weather archives | No | Yes | Web services and downloads | No | No | Not verified | Candidate observed-weather source |
| Peaky Finders (kbaranko) | Open-source application | Yes (ISO load) | Yes (temperature) | Pyiso; Dark Sky API | Yes (Plotly Dash) | Yes (day-ahead) | Plotly Dash, Pyiso | Load + temperature + dashboard |
| GridCast (nzouh) | Open-source forecasting project | Yes (EIA, ERCOT) | Yes (Open-Meteo, GFS) | Data pulls from EIA, ERCOT, Open-Meteo | Yes | Yes (core) | PyTorch Forecasting, TFT | EIA + Open-Meteo workflow; forecasting-centric |
| DemandCast (Open Energy Transition) | Open-source ETL and ML project | Yes (hourly demand) | Yes | Public sources via ETL scripts | Not verified | Yes (ML) | Python, MkDocs, GitHub Actions | ETL for demand + weather |
| Forecasting_Electricity_Weather (alcheng10) | Kaggle-based notebooks | Yes (e.g., PJME) | Weather notebook | Not verified (Kaggle) | No verified | Yes | Not verified | Demand/weather analysis; no pipeline verified |

Table 1. Comparison of reviewed platforms and projects

# 9. Proposed Project Context

The Electricity Demand Monitor is a monitoring-first system. Planned workflow: Data Sources / APIs → Python Data Collection and ETL → Data Cleaning and Validation → SQL Server → Data Analysis → Dashboard → Optional Forecasting Component. SQL Server is the main database; a data warehouse is not part of the current design and could only be considered as a possible future extension if the data volume and requirements justify it.

| Status | Component |
| --- | --- |
| Confirmed core | Electricity demand/load monitoring; weather comparison; Python; SQL Server; dashboard; data collection and ETL |
| Potential / optional | Forecasting; machine learning; additional analytical features |

Table 2. Confirmed and optional components of the Electricity Demand Monitor

Optional components are listed only as possible extensions and are not presented as confirmed implementation. Their inclusion depends on project requirements, data availability, and implementation feasibility.

# 10. Conclusion

This review shows that reliable electricity and weather data are available from official and well-documented sources, and that open-source projects already demonstrate dashboards and forecasting built on such data. Their documentation also shows that value depends on the quality of the data underneath: consistent timestamps, aligned locations, standardized units, and traceable sources. A monitoring system built on proper ETL, validation, and SQL Server storage, with clear visualization of demand alongside temperature, can provide a useful and maintainable foundation. Forecasting remains an open, optional extension that could be added later if data and time allow.

# References

Baranko, K. (2020). Peaky Finders: ISO peak load forecasting application [Computer software]. GitHub. https://github.com/kbaranko/peaky-finders

Electricity Maps. (n.d.). Electricity Maps. Retrieved October 5, 2026, from https://www.electricitymaps.com/

ENTSO-E. (n.d.). ENTSO-E Transparency Platform. Retrieved October 5, 2026, from https://transparency.entsoe.eu/

Hippert, H. S., Pedreira, C. E., & Souza, R. C. (2001). Neural networks for short-term load forecasting: A review and evaluation. IEEE Transactions on Power Systems, 16(1), 44–55. https://doi.org/10.1109/59.910780

Hong, T., & Fan, S. (2016). Probabilistic electric load forecasting: A tutorial review. International Journal of Forecasting, 32(3), 914–938. https://doi.org/10.1016/j.ijforecast.2015.11.011

Hyndman, R. J., & Athanasopoulos, G. (2021). Forecasting: Principles and practice (3rd ed.). OTexts. https://otexts.com/fpp3/

LSB-dev. (n.d.). Publicly available datasets for electric load forecasting [Data set catalog]. GitHub. https://github.com/LSB-dev/Publicly-Available-Datasets-For-Electric-Load-Forecasting

National Centers for Environmental Information. (n.d.). NCEI home page. National Oceanic and Atmospheric Administration. Retrieved October 5, 2026, from https://www.ncei.noaa.gov/

Nzouh. (n.d.). GridCast [Computer software]. GitHub. https://github.com/nzouh/GridCast

Open Energy Transition. (n.d.). DemandCast: Global hourly electricity demand forecasting [Computer software]. GitHub. https://github.com/open-energy-transition/demandcast

Open-Meteo. (n.d.). Historical weather API documentation. Retrieved October 5, 2026, from https://open-meteo.com/en/docs/historical-weather-api

Red Eléctrica / Redeia. (n.d.). REData API. Retrieved October 5, 2026, from https://www.ree.es/en/apidatos

U.S. Energy Information Administration. (n.d.-a). EIA Open Data. Retrieved October 5, 2026, from https://www.eia.gov/opendata/

U.S. Energy Information Administration. (n.d.-b). Hourly Electric Grid Monitor. Retrieved October 5, 2026, from https://www.eia.gov/electricity/gridmonitor/

alcheng10. (n.d.). Forecasting_Electricity_Weather [Computer software]. GitHub. https://github.com/alcheng10/Forecasting_Electricity_Weather